Wiring ERIS A30 with U-Link 2.0 boards ( Untested – 4 wires)

- Insert the battery ONLY after all the wiring and settings have been done

- Keep power down activation active ,

- To maximize battery life on the PE(TX), it is recommended that the Auto Close function remains enabled.

- Make sure the photocells are aligned by following the specific alignment paragraph on the full instruction

- Only remove the link between 70-72 on the control board and leave the others in place (70-71 & 70-73)

- On control boards with an LCD display :

- Go to LOGIC menu and set 1 SAFE = 00 ( active in both opening and closing ) Press OK to confirm

- Go to LOGIC menu and set 1 SAFE = 04 ( active in closing only ) press OK to confirm.

- On control boards without LCD display:

- For Photocell in active in opening and closing no change in dipswitch .