Activate PE beams in Closing only with tested wiring  :

- Make sure the photocells are aligned

- Only remove the link between 70-72 on the control board and leave the others in place (70-71 & 70-73)

- On control boards without LCD display, flick the dip-switch 3 & 4  to UP position  (IO Config ) ) and press the button S2 for more than 5sec to confirm